Mar 25, 2019

# Servo Motor cONTROLLER

_ธีรเชษฐ์  สูรพันธุ์_______ | LE484 Micro APP | Supachai Vorapojpisut

Second project is to create a firmware for a servo motor controller supporting multiple interfaces.

| Task | NOTE | DONE |
| --- | --- | --- |
| Create an MDK-ARM project with STM32CubeMX - Enable SYS > Serial Wire Viewer - HCLK at 168 MHz - EXTI for SW1 - GPIO input for DIPSW1-6 - PDM output for pin PA8 - Input capture for pin PB11 - Analog output for pin PA4 - Analog input for pin PB0 - UART TX/RX pins for pins PD8/PD9 |  |  |
| Write a function for DIPSW1 to generate PDM (pulse duration modulation) signal - Use pin PA8 (TIM1 channel 1) - Generate frequency 500 Hz | Pin #3 of CN1 |  |
| Write a function for DIPSW2 to capture pulse width - Use pin PB11 (TIM2 channel 4) - Return value with 1 sec resolution | Pin #10 of CN1, use polarity both edges Read value of 985 = 985 sec |  |
| Write a function for DIPSW3 to generate voltage - Use pin PA4 (DAC_OUT1) - Generate 1.5 Volt | (1.5/3.3)*4096 = 1862 |  |
| Write a function for DIPSW4 to read voltage - Use pin PB0 (ADC1/2 channel 8) - Return value in voltage | Connect PA4 (at 1.5V) to ADC (pin#13 of CN1) , read 1859 = 1.5V |  |
| Write a function for DIPSW5 to send UART messages - Use pin PD8 (USART3_TX) - Configure 9600,8,n,1 - Use interrupt-driven HAL functions - Print “Hello, world” text | Use SW2 instead, connect Pin #8 of CN4 |  |
| Write a function for DIPSW6 to receive UART messages - Use pin PD9 (USART3_RX) - Configure 9600,8,n,1 - Use interrupt-driven HAL functions - Return number of received characters | Use SW3 instead, connect Pin #7 of CN4 |  |
| Write code for main loop - Use SW1 status to execute functions - Call functions based on status of DIPSW1-6 - Print returned value |  |  |